3D Post-Processing Simulation and MQTT Integration Result Explanation

1. Overview of the Simulation Model

This simulation represents the post-processing workflow of additive manufacturing (AM) parts after the printing stage. Two independent printers generate builds, and all produced parts move through shared post-processing stations.

The implemented model combines:

- Discrete-event simulation

- Process-level energy estimation

- Operator workload estimation

- Part-level tracking

- Shared manufacturing resources

- MQTT-based industrial IoT communication

- JSON-based real-time data publishing

The objective of the model is to analyze:

- Energy consumption

- Operator workload

- Process flow behavior

- Post-processing routing

- Manufacturing traceability

- Sustainability-related metrics

- Digital Twin style data communication

2. Overall Process Flow

The simulated manufacturing workflow follows this sequence:

Printing
   ↓
De-powdering
   ├── Blasting (80%)
   └── Manual Cleaning (20%)
   ↓
Finishing
   ├── Surfacing + Coloring
   └── Vapor Smoothing

Process Explanation

- Parts are first produced by two independent printers.

- After printing, parts move to de-powdering operations.

- Most parts (80%) are routed to blasting.

- Remaining parts (20%) are routed to manual cleaning.

- After cleaning, parts continue to finishing operations.

- Parts are then routed either to:

  - Surfacing + Coloring

  - Vapor Smoothing

3. Printer and Build Identification

The simulation uses SourceID and BuildID to track production sources.

| Parameter | Meaning |
| --- | --- |
| SourceID = 1 | Printer 1 |
| SourceID = 2 | Printer 2 |
| BuildID = 1 | First build from Printer 1 |
| BuildID = 1000 | First build from Printer 2 |

Example

- Printer 1 generates BuildID = 1, 2, 3, …

- Printer 2 generates BuildID = 1000, 1001, 1002, …

This enables:

- Printer-level traceability

- Build-level tracking

- Comparison between production lines

4. Part Routing Logic

Each build contains 5 parts.

The routing logic is defined as:

- 80% of parts → Blasting

- 20% of parts → Manual Cleaning

Printer 1 Example

| PartID | Route |
| --- | --- |
| 1–4 | Blasting |
| 5 | Manual Cleaning |

Printer 2 Example

| PartID | Route |
| --- | --- |
| 6–9 | Blasting |
| 10 | Manual Cleaning |

This routing behavior represents realistic industrial post-processing operations where:

- Automated cleaning handles most parts

- Manual cleaning handles selected parts requiring manual intervention

5. Simulink Routing Result

The simulation output confirms the routing logic.

Blasting Output

PartID:

1, 2, 3, 4, 6, 7, 8, 9

Total:

8 parts

Manual Cleaning Output

PartID:

5, 10

Total:

2 parts

Surfacing Output

PartID:

1, 2, 3, 5, 6, 7, 8, 9

Total:

8 parts

Coloring Output

PartID:

1, 2, 3, 5, 6, 7, 8, 9

Total:

8 parts

Vapor Smoothing Output

PartID:

4, 10

Total:

2 parts

6. Important Routing Observation

The finishing route is independent from the depowdering route.

Examples:

- PartID 4 goes through Blasting and later Vapor smoothing

- PartID 5 goes through Manual Cleaning and later Surfacing and Coloring

This confirms that the routing logic is dynamically controlled at entity level.

7. Process-Level Explanation

7.1 Blasting Process

Blasting is used for automated de-powdering.

Process Logic

- Processing time depends on part volume

- Larger parts require more blasting time

- Energy consumption increases with processing duration

Example Results

| PartID | Volume | Process Time | Energy |
| --- | --- | --- | --- |
| 1 | 5 L | 5 min | 0.417 kWh |
| 4 | 8 L | 8 min | 0.667 kWh |
| 9 | 13 L | 13 min | 1.083 kWh |

Interpretation

The blasting process demonstrates:

- Volume-dependent processing

- Moderate machine energy consumption

- Low operator involvement

7.2 Manual Cleaning Process

Manual cleaning is a labor-intensive process.

Process Logic

- No machine power is used

- Operator performs the cleaning manually

- Operator time equals process time

Example Results

| PartID | Process Time | Operator Time | Energy |
| --- | --- | --- | --- |
| 5 | 27 min | 27 min | 0 kWh |
| 10 | 42 min | 42 min | 0 kWh |

Interpretation

Manual cleaning demonstrates:

- Zero machine energy consumption

- High operator workload

- Strong dependence on human labor

This process is important for labor-cost estimation.

7.3 Surfacing Process

Surfacing improves part surface quality before final finishing.

Process Logic

- Moderate machine power usage

- Short process duration

- Low operator handling time

Example Results

| PartID | Process Time | Energy |
| --- | --- | --- |
| 1 | 5 min | 0.417 kWh |
| 5 | 9 min | 0.750 kWh |
| 9 | 13 min | 1.083 kWh |

Interpretation

Surfacing demonstrates:

- Controlled finishing energy

- Surface preparation functionality

- Low operational complexity

7.4 Coloring Process

Coloring is one of the major finishing operations.

Process Logic

- Processing duration increases with volume

- Energy consumption increases linearly with process time

- Operator involvement is moderate

Example Results

| PartID | Process Time | Energy |
| --- | --- | --- |
| 1 | 60 min | 4.0 kWh |
| 5 | 108 min | 7.2 kWh |
| 9 | 156 min | 10.4 kWh |

Interpretation

Coloring demonstrates:

- Significant energy consumption

- Strong volume dependency

- Long finishing duration

7.5 Vapor Smoothing Process

Vapor smoothing is the most energy-intensive finishing operation.

Process Logic

- Long processing duration

- High machine power

- Large energy consumption

Example Results

| PartID | Volume | Process Time | Energy |
| --- | --- | --- | --- |
| 4 | 8 L | 96 min | 24 kWh |
| 10 | 14 L | 168 min | 42 kWh |

Interpretation

Vapor smoothing demonstrates:

- Extremely high energy consumption

- Long processing cycles

- Heavy resource usage

This process becomes a major contributor to total manufacturing energy.

8. Total Energy Accumulation

The simulation accumulates energy across all stages.

Example

| Stage | Energy |
| --- | --- |
| Printing | 40 kWh |
| Blasting | +0.417 |
| Surfacing | +0.417 |
| Coloring | +4.0 |
| Final Total | 44.833 kWh |

This enables:

- Total manufacturing energy estimation

- Part-level energy tracking

- Sustainability analysis

- Life Cycle Inventory (LCI) modeling

9. Shared Manufacturing Resources

The simulation uses common shared post-processing stations.

Examples

- Shared blasting station

- Shared manual cleaning station

- Shared surfacing station

- Shared coloring station

- Shared vapor smoothing station

This creates realistic industrial behavior such as:

- Queue formation

- Shared resource utilization

- Production bottlenecks

- Parallel production flow

10. MQTT and JSON-Based IoT Integration

Each completed process publishes a separate MQTT JSON message.

MQTT Topics

- tamk/postprocess/blasting

- tamk/postprocess/manual_cleaning

- tamk/postprocess/surfacing

- tamk/postprocess/coloring

- tamk/postprocess/vapor

Example JSON Payload

{
  "process":"Vapor",
  "source_id":2,
  "build_id":1000,
  "part_id":10,
  "volume":14,
  "process_time_min":168,
  "energy_kwh":42.000,
  "total_energy_kwh":90.000,
  "finish_type":2
}

Published Information

The JSON payload contains:

- Process name

- Source ID

- Build ID

- Part ID

- Part volume

- Process duration

- Process energy consumption

- Total cumulative energy

- Finish route type

This enables:

- Real-time industrial IoT communication

- Entity-level process monitoring

- MQTT-based Digital Twin integration

- Structured manufacturing data streaming

11. Industrial Significance of the Model

The model supports:

- Digital Twin development

- Energy monitoring

- Production optimization

- Operator workload analysis

- Sustainability assessment

- Life Cycle Assessment (LCA)

- Manufacturing traceability

- IoT-enabled smart manufacturing

The simulation also enables:

- Part-level tracking

- Build-level tracking

- Process-level energy analysis

- Comparative route evaluation

- Real-time process data streaming

12. Key Findings

Main Findings

- Vapor smoothing consumes the highest energy.

- Manual cleaning consumes no machine energy but requires high operator time.

- Coloring duration increases significantly for larger parts.

- Shared post-processing stations create realistic production behavior.

- Process-level energy accumulation enables part-level sustainability analysis.

- Different finishing routes create different energy footprints.

- MQTT integration successfully enables structured real-time process communication.

13. Current Limitation

Currently, MQTT publishing is performed using MATLAB mqttclient scripts based on simulation result values.

The next implementation step is to automate direct MQTT publishing from Simulink entities and process completion events.

14. Conclusion

This implementation demonstrates how discrete-event simulation combined with process-level energy modeling and MQTT-based industrial IoT communication can support Digital Twin development and Life Cycle Inventory analysis for additive manufacturing post-processing systems.

The model successfully integrates:

- Manufacturing flow simulation

- Energy estimation

- Operator workload estimation

- Shared resource behavior

- Part-level traceability

- MQTT communication

- JSON-based IoT data streaming

- Sustainability-oriented analysis

The generated results can later be integrated into:

- IoT platforms

- Digital manufacturing dashboards

- LCI/LCA frameworks

- Smart manufacturing systems

- Industrial Digital Twin environments